PROJECT  SPECIFICATION

v.3 2007-07-18

Abstract: This document serves as a project specification in structuring the proposed approaches, limiting the scope, stating the assumptions, and giving when identified some of the needed sub-models, optimization principles and data .I describes the industrial, societal and research deliverables, including a functional description of the emissions control box. It also gives the basic generic models for some of the functionalities, as well as some key emission data necessary to formalize the criterion functions to be optimized .It contrasts the proposed research and focus with the many available housing energy models, with a focus on dynamics and optimization aiming at sustained emissions reductions. Some open issues identified sofars are indicated aiming mostly at reducing the options.

TABLE OF CONTENTS:

1-HIGHLIGHTS OF RESEARCH CHALLENGES AND DELIVERABLES

2-PROJECT SCOPE and OBJECTIVES

3-DELIVERABLES

4-MODEL TYPES AND RESEARCH FOCUS

5-CATEGORIES AND DYNAMICS OF LOAD CONTROL VARIABLES

6-PHYSICAL and USAGE FLOWS

7-OVERALL INTERACTIONASSUMPTIONS AND EQUATIONS

8-EQUIPMENT ALTERNATIVES w.r.t ENERGY SOURCES

9-EMISSIONS CONTROL BOX

10- EMISSION PERFORMANCE MEASURES

11-EMISSIONS ANALYSIS

12-EMISSION AND COST OPTIMIZATION CRITERIA

13-OPTIMIZATION ALGORITHMIC APPROACHES IN THE PROJECT

14-OPTIMIZATION CONSTRAINTS

15-RANDOM FACTORS

16-SOME INNOVATIVE TECHNOLOGIES AND OTHER RELATED RESEARCH AREAS

17-REFERENCES

APPENDIX I: CARBON INTENSITY FACTORS OF FUELS

APPENDIX II:  SURVEY OF UK HOME ENERGY AND EMISSIONS MODELS

Separate document: INTERNATIONAL SURVEY OF HOME ENERGY AND EMISSIONS MODELS

Artwork: (13 Figures, 8 Tables)

- HIGHLIGHTS OF RESEARCH CHALLENGES AND DELIVERABLES

  - Background

According to the UN, the UK had in 1990 CO2 emissions of 558770 Mtons, and 504575 Mtons in 2004 that is a 14, 3 % reduction. This project focuses on giving model and computational tools support to further reduce emissions from housing. Heating in housing is approximately a quarter of the national total!

Figure 1: Research challenges and deliverables

1.2. Research challenges

While most research till now has studied energy consumption ,and derived emissions, from static average conditions of a house , with averaging assumptions on is environment , this project will focus primarily on the CO2 emissions using a dynamic time dependent model (Figure 1).

While most research and models sofar produce consequences from a set of assumptions, this project will determine time-dependent load levels which minimize absolute emissions over 1 year ,and over 1 average day, respectively

While some research looks at the cost of energy consumed in a house, the proposed optimization of emissions will be carried out with either a constraint on capital budget expenditure (investment and operating costs for emissions prone fixed equipments) , or on the derived costs linked to those optima which have been determined

While most research assumes no time dependent behaviour of occupants, or assumes for them and appliances an average usage, this project will explicitely take different time-dependent service demands by separate occupants into account, adding a level of sophistication and pointing at emissions irresponsible behaviours

While most research assumes a fixed default configuration of fixed equipments and appliances, in this project some level of combinatorics will be explored so that equipment alternatives (by size, nature, energy source) can be explored

While projections from static models allow for consequence calculations, in this project the optimizations will be performed with key critical parameters as parameters so that heir effect not just on projections, but on optimal designs can be established

The following Sections will justify the initial project specifications and compare them with state of the art.

2. PROJECT SCOPE and OBJECTIVES

2.0 Overall Objective

The main purpose of the project is to minimize emissions in a single existing house subject to constraints including costs via the time dependent optimization of heating, ventilation and lighting usage. The assumed heating sources are gas and electricity as they dominate the existing housing stock .The models and optimization tools should be extensible to support housing refurbishment, or new energy sources and home designs.

2.1 Specific scope and assumptions

They are to:

- Emissions optimization goal :optimize emissions from a typical single  existing UK house ,parameters of which can be altered, taking an integrated approach of all emissions related processes ; the optimization is by determining the load controls for heating ,  ventilation and lighting sources

- Constraints and economics : optimization will be carried out subject to constraints on thermal comfort with cost consequences ;

- Housing type scope : research confined to an existing UK housing , which sets the requirement on  defining UK house parameters and UK usage thereof

- Consumption types and scope : regarding the consumption , it is for most energy using products, with emphasis on heating, cooling, ventilation but also on major  appliances ; the research in particular will  take usage patterns by occupants into account to the extent they affect energy demand ;

- Emission sources : emissions from ALL energy sources and main appliances (heating, hot water, oven, lighting,  computer, other   power used or produced etc),

6    Heating source types: regarding heat sources, the focus types are the conventional types found in the existing housing stock     (see assumption 1); the primary heat sources considered will be gas and electricity as they represent the most common combination in he existing housing stock

- Time horizons of dynamic analysis : focus on the dynamics of mix of fuels and  emissions over 1 year cycle and on 1 day average cycle

- Variability factors : account for average variability over days, weather ,and occupancy , and incorporate these

- Control designs : design time and temperature dependent controls of loads meeting stated optimization goals ; low level stabilizing controls against target values will not be dealt with ; schedule the fuels where there is a choice amongst multiple fuels

- Engineering deliverable : will be the coordination unit in the home which orchestrates the minimal total emissions by sending dynamic load control information to existing or future system or device controllers already produced by industry

- Benchmarking : compare the results with other modelling tools such as heat loss BRE , SAP “Standard assessment procedure” , CI Carbon Index [93],  BREDEM and others [87]

- Energy consumption calculation : the project is limited to optimizing energy in use rather than embodied energy,  but wherever possible the research will factor in whole life analysis (what it costs to produce the materials to be used in new equipment, products etc)

- Regulatory impact : consider how the results of the research can be interpreted into regulatory terms (Building regulations)

- Software , algorithmic and interface extensibility : the software tools, algorithms and supporting data bases will be designed to allow to potential extensions for specific refurbishment approaches , as well as to determine emissions reduction in new builds

Regarding the first point, focus is on existing UK houses as they represent the largest stock and opportunity for emissions savings. This will allow estimating the implications on the existing home stock, using impact factors found such as those found in DTI’s study [83].  Is it also pointed out that new energy sources will not be focus, although it will be possible to model them and optimize in the same way with different assumptions. The housing architecture though should remain unchanged.

Regarding the heating sources , as focus is on common stock houses, the project will not consider initially new home energy sources such as heat pumps , solar thermal, etc ,although by later adding these variants  to the tools to be developped  this extension should be possible,  akin a normal house refurbishment process .

Regarding the evolution of regulatory instruments, including SAP ratings, this is important as critique has been raised already [85, 90] showing that if CO2 emissions are to be reduced significantly, these procedures must evolve. Also the underlying models like ESP-r use quite old equipment data focussing on energy consumption only and not on emissions .This justifies the reduced dependency in this project upon SAP assumptions and policies.

As to energy sources in relation to energy consuming equipments , the project will be unique in handling combinatorial alternatives and thus adding many more choices than in existing building models .Nevertheless, some reduced choices will be made on energy sources, e.g. not including initially solar , wood ,home based wind power ,under floor heating  ,just to reduce the combinations ; focus will instead be on gas and  electricity .

Thus this Initial project specification document has identified or selected models, data or information, based on these requirements and project phases, aiming at a fast start-up with innovative research potential.

  - DELIVERABLES

3.1 Industry deliverables

Industry would receive:

-a toolkit made of software, algorithms, and some data bases, whereby to design:

a) either a coordination box, call it “emissions reduction box” which will receive needed sensor inputs, and which orchestrate the emissions-reducing loads on a range of home equipment and main appliances; this would correspond to a closed loop emissions reducing control; in that case industry will have to build drivers and interfaces to the needed sensors, as well as the communications links between equipment controllers and the emissions reduction box

b) or the toolkit whereby to load into equipment specific controller’s ROM  ( such as boiler control) default load controls aiming at reducing the emissions ,with time and existing reduced set of sensor inputs ; this would correspond to an open loop emissions reducing control

- emissions impact results from the optimization from an existing home stock , with possibilities to plug in industry specific data and  equipments ; this is essential to be able to market emissions reducing features o home owners or builders alike taking most of the other effects in the house into account

Industry should be able from the determined optimal load patterns over 1 year or 1 day, to adjust the control laws in their equipments taking through this step indirect account on overall house-level emissions-focussed consequences ; this will include substitution , interaction and pre-emptive effects

3.2. Society wide deliverables

Even if a coordinated emissions reduction optimization can only yield say an emissions gain of say -2 % to +5 % on a day (owing to seasonal fluctuations and other random elements), and a sustained average emissions reduction of 2 % over a full year, such reductions still represent an essential step in the right directions where it is hardest: the dominating home stock.

There is another society wide benefit, that is to allow to use such tools and methods in environmental and energy services  , to help determine, in specific cases , which refurbishments to make, how to establish incentive schemes with realistic goals, etc . One idea is for example the certification for reasonable CO2 emissions of existing housing at the occasion of each new long term rental or sale (as in France since 01 November 2006).

3.3. Research deliverables

Even if the relative gain achieved by optimization, due to all uncertainties, would only be small, the research should allow to estimate whether priority in emissions control policies should include or not dynamic effects and should allow to size qualitatively the relative importance thereof

By comparison with regulatory instruments in force today, the research should allow to pinpoint needed changes if it turns out that emissions cycles are different from energy consumption cycles, and/or if dynamics should be included in their indicators

For the research community, this project allows altogether to develop optimization framework in uncertain complex environments such as housing, and determine wireless or other sensing schemes to better distribute control systems in them

4. MODEL TYPES AND RESEARCH FOCUS

a) Almost all models identified in the survey below (List of UK models in Appendix II,  References in Section 17 and Addendum with international survey of models ) share the following  common features in that they :

-rely on one hand on some simplified static physical models especially for emissions, heat balance and dissipation or absorption

-rely on some experimental rules

-are fragmented to apply in rather large time-intervals and/or space volumes only (such as a whole night, or a whole room)

-have location, solar irradiation, seasonal, materials and other choices as parameters

The consequence is that, from a modelling point of view, almost all models are built with:

-either explicit formulas, or nested formulas valid within a specified time/space/configuration context

-or nested static formulas with integrated model structure provided in almost all cases by spreadsheet type tools

There are only very few cases where other techniques have been used, such as neural networks, stochastic control and alike.

b) Assumptions like the ones made above in a) are valid ones for the proposed project goal (Section 2), but the analysis of emissions and their optimization cannot rely on just context dependent static formulas or static integrated models:

- one main reason is that emissions are the integral over long periods of time of time-dependent emissions under different heating/cooling/passive heating regimes for a house. Thus at a minimum, emissions should be determined per day and not by sun position and related hour of the day:

- another aspect is that none of the models as surveyed take housing usage into account, meaning that the people’s movements and use of the rooms is not accounted for, so savings and excess energy are left to local home controls to regulate short term only. The domestic heating compliance guide is however a good read, and hints at the importance of zoning.

c) As an example of state-of-the-art work there is OpenTherm. All the industrial members of this project are members of the Opentherm association www.opentherm.og .Opentherm is a modulating condensing boiler control system that has shown , in controlled tests , up to 14 % energy efficiency gains , with a similar reduction in CO2 emissions .Opentherm does not have a standard RF interface defined. It is a point-to-point rather than a bus system and cannot operate with multiple sensors and/or ad hoc networking .The proposed sysem in this project will overcome these shortcomings .Furthermore , the effetcs of random events that interfere with the Opentherm environment have not , to our knowledbe, been investigated before .Similarmy control systems (often more than a timer/programmer and a thermostat) generally have no knowledge of what the total load demands are on the boiler ; in this project to the contrary the emission control box will coordinate the loads for minimum emissions .

d) The proposed research should focus on long term emission optimization in presence of parametric patterns of usage, and take lead/lag dynamics into explicit account as well as the joint emissions from all rooms .Should be used MinMax dynamic optimization possibly with some stochastics, under changing general configurations and distinctive parameters.

In the references, optimization has been criticized for its high sensitivity to assumptions and deviations from these assumptions; this is a valid argument, but MinMax dynamic optimization is rather aiming at determining fundamental changes to housing equipment and its usage rather than aiming at approximating accurately measured data.

Furthermore, as indicated in Section 1, this optimization must be under cost constraints .The costs are a yet to be determined mix of fuels costs, equipment write-down costs (including house controls), and emissions costs.

5. CATEGORIES AND DYNAMICS OF LOAD CONTROL VARIABLES

5.1. Relevant control categories

The control variables are those time-dependent commands to be applied to heating, ventilation and lighting to ensure minimal emissions in a house.

From the high level emissions controls policies (such as the Intergovernmental Panel on Climate change (IPCC) ) , a taxonomy of control variables to be possibly used, and narrowed down to those relevant to a house, can be suggested:

a) Control variables with impact on the short term emissions optimization

a1:-Alternative heating sources and fuels

a2:-Alternative cooling sources and sources

a3:-Heat and cooling distribution leakage inside the home

a4:-Dynamic efficient and natural lighting

a5:-Active and passive solar energy use

b) Control variables with impact on the longer term emissions optimization

-Home based recycling

-Home based carbon capture

-Use of new batteries

However, because of project scope and assumptions (see Section 2), from a) and b) again only a subset of control variable categories will be the focus of the research, namely:

a3:-Heat and cooling distribution leakage inside the home

a4:-Dynamic efficient and natural lighting

5.2. Quantization of load control variables

As the project does not deal with the engineering design or the stabilization controllers of the many equipment and appliances in the house, but in consideration that it will design dynamic emission profiles, the pragmatic approach is, for each control device to have finite usage-related discrete values transitions (see Figure 2):

-Level 0: Off

-Level 1: Low output level

-Level 2: Medium output level

-Level 3: Peak output level

These output levels also corresponding to what the building industry calls “loads” The goal of the emission optimization is then jointly to minimize emissions by selecting over time the changes to be made to all controls amongst the four approximated control levels.

Figure 2: Optimized time dependent control level transitions

The industrial partners engaged in the project should then be able, for their specific proprietary equipment designs, to map back the optimal control paths with the set-point values and variables to be used inside their proprietary control systems. As an example, while Danfoss Randall in [88] [89] shows that benefits from high efficiency boilers are lost if controls are not updated to optimize boiler performance , the source is stated to be in the high variability in the return water temperature ( 35 degrees C to 60 degrees C oscillations ) caused by other boiler control thermostats than chrono-proportional ones (with 6 updates/h) .By optimizing for emissions the set-point boiler and room temperature values over time, further gains are expected , while advanced controls would give very good comfort (room temperature oscillations of less than 0,3 degrees C) .

5.3. Equipment control dynamics

Very few data have been identified sofar giving for each control device, its dynamic characteristics. While the control law implemented is specific to each device , some time response information is still needed .Those exceptions with time response data are for different types of  thermostats [88] [89] [90]  , giving output water set-point temperature . The proportionate based weight K is assumed to be equal to K=1 and the integral action time constant I= 300 seconds.

Weather compensation control law examples are also given in [90], giving the water supply temperature set-point vs. outside temperature.

6. PHYSICAL and USAGE FLOWS

The control variables being loads onto a diversity of equipments and appliances , some with own local controls , will impact jointly the emissions from the house while in use .These emissions in this project are the result of the time-dependent  interaction between four key physical and usage flows as depicted in Table 1 :

| Physical or usage flow medium | Common prime effect |
| --- | --- |
| Indoor air | Room temperature and comfort |
| Hot water | DHW (Domestic hot water) |
| Light | Lighting |
| Inhabitants | Home and appliances utilization |

Table 1: Physical flows and usage

These flows interact again because of heat generation , dissipation , thermal transfer , air circulation and ventilation , activation of lighting , activation of appliances, and human activity , which are all the processes leading to changes in instantaneous emissions. The levels of instantaneous emissions are again dependent upon the efficiencies of all these processes, in that inefficiencies will force a higher flow of air, hot water, light and/or changed usage patterns. Whereas the analysis of these interacting processes is related to energy consumption analysis, the focus on emissions is quite different as absolute emission levels are of concern instead of energy supply flows .In this way, even the dynamics, the time constants, the uncertainties and the optimization algorithms are quite different and thus unique to this project.

Figure 3: Flows, interaction processes, and the emissions optimization affecting the dynamic controls

In terms of energy types and consumption driving the processes, the focus will be on gas and electricity as explained in Sections 2 and 7.1. The gas will drive the boilers and possibly some appliances .The electricity will drive appliances (but not heating) Electrical consumption in a household is missing from the initial report paper. The load from white goods and luminaries in the home will be the most significant electricity consumers (compared to a heating system).  Additionally these all provide secondary heat sources to the home, especially fridge/freezers and TV’s that have a high percentage of “on” time.  Plus the “standby” of much electrical equipment these days can also add to a significant consumption during periods where a home is unoccupied or unused (during the night for example) and is certainly where a lot of current carbon reduction work is targeting. In the emission quantities to be optimized are not included those emissions who happened for the production of the two types of energy used in the home ; count only from the energy sources, the emissions from energy consumed in the house, with of course consideration for efficiencies .

Unique to this project is also the explicit use of flows of human activity ;the components thereof are the service needs of the individuals when using the house ; each inhabitant has a time profile of needs in which each service has to be provisioned ; the number of inhabitants may be determined by volume in the house using volumetric people’s detectors (see Section 16.1)  .This “service provisioning concept” is also more in line with state of the art business concepts than just usage proportional to the number of people resident at that home’s address . In this way changes in personal behaviour may result in direct emissions reductions, and diverse behaves are still possible.

Figure 4: Example of service provisioning and service choices for the inhabitants

7. OVERALL INTERACTION ASSUMPTIONS AND EQUATIONS

This Section 7 addresses the main sub models for an initial project focus following Sections 1-3.The goals are to reach a compromise between he following aspects:

-verified models accepted by industry or in standards

-applicability suited for use with the more advanced techniques summarized in Section 13

-generic aspects so that a diversity of add-on local closed-loop control devices can be added, as well as alternative or specific heat sources and appliances

-applicability for use when dynamics are studied as well as multi-room environments

We colour label the quantities considered:

-Brown is geometry or time interval or context related

-Green is appliance related

-Yellow is usage related

-Red is controls

-Blue are state variables

7.1. Base house configuration

For the house type, the assumption is made to select the 89 M2 stock semi-detached house layouts; representing in UK about 80 % of the legacy housing; see Table 2 for specifications. This reference case is the one used for the BRE Domestic energy model [94].

Any analysis will require the prior specification of at least:

- Boiler location and room

- Distribution of hot water

- Distribution of cool fluids

- Ventilation inlets and outlet(s)

- Window locations and sizes

- Door locations and sizes

- Appliance types

- Appliance locations

- Density map of human activity (number of people and time density)

| 2010 data: -Semi-detached dwelling -Floor surface : 88,8 m2 -wall U value : 1,1 W/m2K (solid walls: 2,1 W/m2K , filled cavity : 0,466 W/m2K) -150 mm loft insulation -Roof U-value : 0,322 W/m2K -Uninsulated solid ground floor U-value : 0,684 W/m2K -draught proofed single glazing window U-value : 4,7 W/m2K -solid wooden door U-value : 3,0 W/m2K -Hot water cylinder insulation jacket thickness: 55 mm - 20 % low energy lighting -gas boiler with SEDBUK efficiency 67 % -cylinder thermostat present -primary pipe work uninsulated -cylinder insulated to the stock average level , equivalent to a 55 mm insulating jacket on a 12 litre cylinder -room thermostat and programmer ,but no TRVs | 1995 data : -Semi-detached dwelling -Floor surface 89 m2 - 3,2 radiator size (living area) -5,5 radiator size (non living area) -8,7 total radiator size -11,7 boiler size -100 mm loft |
| --- | --- |

Table 2 : National housing stock 2010 data [87] and BREDEM Domestic energy model reference data [94] are  on the left  ; to the right  are given from [90] the Type C national housing stock 1995 parameters found in that report ;the issue remains how to recon ciliate the two

As benchmarking indications, are reproduced below the expected benefits of domestic gas wet central heating systems, with filled cavity walls,  that meet CheSS specifications [94] ,for a semi-detached house as specified in Figure 5 (left column) :

|  | Reference case (Fig 5) | CheSS Basic | CheSS Best |
| --- | --- | --- | --- |
| SAP index | 69 | 79 | 83 |
| Carbon index | 5,0 | 5,9 | 6,3 |
| Energy (kWh/y) and % saving on reference | 17,100 | 22 % | 30 % |
| Cost saving GBP/year |  | 57 | 79 |

Table 3: Expected benefits on emissions, energy and costs [94]

7.2 Heat balance

The basic heat balancing principle is to ensure that, in the presence of external fluctuations / usage / leakage, a target temperature is maintained with minimal fluctuations by applying controls to heat, ventilation and lighting sources. The corresponding heat balance equation applies to each volume in the house and is summarized in Figure 5. The heat demand is the heat to generate in each such volume by applying said controls to maintain the target temperature.

Figure 5  : Heat balance model for a given volume V ; the volume contains a heat source , distribution pipes, appliances , radiators , ventilator, and people and their interaction results in an indoor  temperature T(t) at time t ; the airflow is mostly recuperated and walls / windows have thermal inertia ; for details see the equations in Section 7

The assumed general heat balance equation giving heating demand (QH in kWh for a period of time) follows from ventilation losses (QV), transmission losses (QT) minus solar & internal gains (QG) and user based energy contributions (QH):

QH= QV+QT-QG-QH =

0,001*[hrs*V*(T-Tout)*[ah*(qv*(1-qrec) +qinf) + (AV*U)]-F*(qgain+qsol*sgf)*dpm-Nhumans*qhum] – Qdistribution-Qconv-Qenv

where this equation is supposed to be time-dependent over a time interval hrs to the contrary of the referenced models, and:

- 0,001 is the conversion factor for Wh to kWh

- hrs is the time-interval in hours

- V is the internal volume in m3

- T is the indoor temperature at that moment in degrees C

- Tout is the outdoor temperature in degrees C

- ah is the specific heat constant of air or 0,33 W/km3

- qv is the ventilation rate vs. the analyzed volume ,in m3/m3

- qrec is the energy recovered from outgoing ventilation air , in %

- qinf is the infiltration rate vs. the volume considered in m3/m3

- AV is the ratio between the volume considered V and its exterior surface A , in m

- U is the equivalent specific transmission coefficient per unit of exterior surface A ,in W/km2

- F is the heated floor area of the volume considered in m2

- qgain is the internal gain from appliances etc per time period hrs

- qsol is the global solar irradiance per time interval hrs in kWh/m2.hours

- sgf is the solar gain factor

- dpm is the average number of days per month (30,5)

- Nhumans is number of users in the volume in that time interval hrs

- qhum is irradiation per human per hour

- Qdistribution (see below) is heat distribution from pipes between boiler and radiator

- Qconv: convection loss from boiler (see below)

- Qenv : dissipation loss by boiler (see below)

When there is no heating (QH=0) the indoor temperature can be calculated .Likewise the maximum achievable temperature can be calculated at maximum heating output QH=Qhmax:

T= Tout+ [QH+F*(qgain+qsol*sfg)*dpm+Nhumans*qhum]/ [hrs*V*[ah*(qv*(1-qrec) +qinf) + (AV*U]]

7.3 Thermal mass

This is considered a difficult determination, but an approximation is to take the first 3 cm of the heavy construction components (excluding windows, doors, ventilation and infiltration flows):

Tmcon = 0,001*tm*V*(T-Tout)

where :

- Tmcon : thermal mass of the construction’s first 3 cm

- tm is the DIN specific thermal mass per unit of brick building volume tm=18 Wh/m3.K

Taking roofs into account is though a complicated matter due to insulation measures; some data can be found from Rockwool flat ROCKROOF system.

The thermal mass of the water in the radiators is:

Tmrad = 0,001* 1, 16 *(Tad-T) *20 * Pradnom

where:

- 1,16 is the specific heat of water in Wh/kg.K

- Tad is the average radiator temperature in degrees C (range 20-70 degrees C)

- 20 is water volume of a radiator per kW of nominal radiator capacity in litres/kW

- Pradnom is the nominal radiator capacity according to EN 442 in kW

7.4. Non-controlled radiator cool down

This equation is uncontrolled after radiation shut down has been ordered: it gives for Nhuman=0 an approximation of the stationary indoor temperature T after switching of at time t=0:

T(t) =Tout+tmcf*(Tprev-Tout)*exp (-t / [tm*V/1000*PH/(Tprev-Tout))]]

where :

- tmfc is a correction factor in the range 0,95-0,99

- Tprev is the indoor temperature at t=0 prior to switching off radiator

- t is time spent since switch off in degrees C

- tm is the effective 3 cm specific thermal mass of the volume in W.h/m3.K (e.g. 18 for heavy buildings and 12 for light buildings)

- PH = (QH/hrs) is the average heat demand QH calculated at Tprev and Tout,  in kWh

- Hrs is the time interval in hours

7.5 Heat or cold distribution loss

This dissipation from conductors is assumed proportional to the product of the length by the temperature difference; no circulating pump is assumed:

Qdistribution = -qdistri-hot * (Tradh-T)* Lhot –qdistri-cold * (T-Tradc) * Lcold

- qdistri-hot : heat dissipation from heating distribution per unit length and temperature difference in degrees C between Tradh and T

- Tradh : heating radiator fluid  temperature

- Lhot : distribution length for heating radiator in volume V

- Qdistri-cold : cold absorption from cooling distribution per unit length and temperature difference in degrees C between Tradc and T

- Tradc: cold fluid temperature in degrees C

- Lcold: distribution length for cold in volume V

7.6 Air flow

The outgoing airflow is V (qv+qinf), where qinf may be negative if there are airflow losses .It is assumed though that the airflow is circulating and recuperated with resulting energy share recuperated.

7.7. Heat transfer between volumes

Figure 6: Heat transfers between volumes i and j; each volume in turns obeys a heat balance

The references in Section 17 give very little basis for modelling the heat transfer between volumes; the only general formula is:

QH(i,j) = hrs*(T(i)-T(j))*(b(i,j)*qT(I,j)+qinf(i,j)*V(i,j))

Where :

- QH(i,j) is the heat transfer between zones i and j in kWh

- b(i,j) correction factor for transmission losses

- qT(i,j) : specific heat transfer between I and j = A(i,j)*U(i,j)

- A(i,j) : surface between volumes i and j in m2

- U(i,j): specific transmission value eg for internal wall or door between I and j in kWh/K = V(i)*qinf(i,j)

- V(i) reference volume ,usually the smallest of V(i) and V(j)

- qinf(i,j) : specific ventilation rate in m3/m3 relating to V(i) and V(j) is linked by ventilation outlet and ventilation volume is then V(i,j) (approximated by the sum)

The transfer equations above must be adapted to account for the thermal inertia of the walls Tmcon for both volumes  .The above suggests that the use of FEM finite element numerical analysis, both for volume and time segments, is a possible extension to the research.

7.8. Ventilation flow between volumes

The usual flow-in, minus losses, plus new flows, gives the flow out assuming a unidirectional flow in the ventilation and one exhaust point.

7.9 Boiler

The project team has access to the BRE/ESRU boiler model data [90] and to a simulation model for EU wide domestic heating from the EU Eco-Design of Boilers Stakeholder Group (www.ecoboiler.org). The BRE/ESRU Advanced Controls Project  has a working static model of both the controls, boiler and emitter sides of the system; it uses for all calculations the ESP-r model (see Section 17 and [91])  ;this  has already been contributed to by both Danfoss, Honeywell and other controls manufacturers and the boiler industry. The Ecoboiler model is a basic look-up table based Excel spreadsheet simulation, hence the relevant mathematics are easy to access and relatively simple arithmetic rather than containing any calculus.  Some details of boiler types are found from this European project www.ecoboiler.org especially in Task 4 reports. No generic model is given as there are many variants including on the heat exchanger, burners, controls etc.

As no such generic model is given, the project would have to select the central heating specifications from standard alternatives such as those listed in [94]: CheSS-HR3, CheSS-HC3, CheSS-HR4, CheSS-HC4.

This is a solid base for later design of controllers at equipment level (see Section 3) but does not give the dynamics nor the time constants required for time-dependent optimization. Also these two models focus on energy and not on emissions, for which time characteristics are different (see below).

It should also be stressed that variables labelled below as control variables are in general indirect controls as the gas flow, fuel flow etc. ,and are not the actual controls set by the burner control .But as burner dynamics are yet unknown in this project , or may be too specific, the project may not use many actual controls .

Therefore focus is put here on the general boiler heat transfer equations and the derived dynamics, especially those of emissions, so that the optimization can make use of these elements.

The boiler should output a temperature Tb which is the radiator temperature. The controls are the phases of the boiler first (see below) and next gas/oil volumes and speed. In turn the control on the flame nozzle impacts the volume of fuel.

The Stefan-Bolzman formula gives the linkage between convection heat and radiation:

Qrad = A*eps*sigmas*(Tg**4-Tw**4)

This is the same as the total heat transfer through radiation of the burner towards the heat exchanger.

Where:

- Qrad is the radiation heat energy in W

- A is the surface of radiation heat transfer in m2

- eps : the emission factor with 0,2-0,3 for a normal burner and 0,6 for radiation burners

- sigmas : h Stefan-Bolzman constant of 5,67 x 10**(-8) W/m2.K4

- Tg : temperature of gas in degrees K

- Tw : temperature of wall in degrees K

The convection heat transfer depends linearily on the temperature difference:

Qconv = A *alpha* (Tg-Tw)

where:

- alpha: heat transmission coefficient in W/m/K

Boiler convection heat dissipation losses through the envelope are approximated by the Nussel formula:

Qenv = 2, 6 * Aenv * (Tw-Tboiler) ** 1, 25

where:

- Qenv is the loss through the envelope

- Aenv is surface of boiler envelope

- Tw is assumed to be the same as the temperature of the envelope

- Tboiler is temperature inside boiler

The boiler efficiency can be modelled as in [90] as a 4th order polynomial giving the gross efficiency vs. return water temperature for different load % ; the polynomial and its coefficients are found in [90] for three types of boilers : gas condensating, gas non-condensating, oil condensating .

The boiler has strongly non-linear cycles:

-burner on: flue gas losses, radiation, convection and conduction losses through the generator envelope

-burner off: standing losses such as radiation, convection and conduction

-start-stop losses: pre-purge losses, after purge losses/gains, efficiency losses caused by cycling

Figure 7 shows a typical boiler activation cycle with the delays prior to stabilization .Figure 8 shows that the emissions are actually having a different time dependent profile than temperature and of energy consumption, due to dirty burn in the initial start-up. Likewise, preheating the boiler and/or fuel may lead to fewer emissions .Such effects will have to be taken into account in this research. Figure 9 (Source: Danfoss) shows RMT Grade A condensing boiler temperatures over time, highlighting the cycles and the evolutions thereof.

Figure 7: Boiler activation cycle giving temperature vs. time

Figure 8: Boiler emissions vs. time at start-up

Figure 9: RMT grade A condensing boiler dynamics 27/4/2006 5Source and 5C): Danfoss)

Need to be clarified modelling assumptions as to the effect of the boiler controls as this research cannot deal with burner modelling of the inside boiler temperature, with heat exchanger nature and characteristics, and obviously with the effects of the surrounding volume to the boiler. However, boiler controls effects over time will be taken into account; Figure 10 shows how the combined non-linear controls result in time dependent temperature adjustment profiles

Figure 10: Effects of boiler controls to temperature adjustments

The project shall not address the special aspects of wood furnaces.

7.10 Pump for heat distribution

There are different views as to whether heat distribution pumps are included or not into the boiler characteristics .It will here initially be assumed that the pump is embedded with the boiler .In [90] are found from Grundfoss pump characteristics giving hPa vs. system flow in kg/s.

7.11. Hot water usage

The hot water usage is closely linked to the time interval, time of the day and number of users.

The dissipation from pipes is discussed above.

7.12 Cold water usage

The cold water usage is closely linked to the time interval, time of the day and number of users.

The heat absorption by pipes is discussed above. Cooling water circulating at 5 degrees Celsius creates a condensation which leads to energy loss; at 7 degrees the problem disappears. No model has ye been found for this condensation.

7.13 Lighting

Lighting models exist and some are mentioned in the surveys below. They ultimately depend on the lighting and usage configuration assumptions yet to be made.

7.14 Appliances

There is an enormous diversity and amount of information on appliances’ energy consumption , usage, labelling and emissions (see policy level discussions on UK Government’s “market transformation” Web site ; Environmental change Institute, Oxford)  .Some simplifying assumptions have to be made to get a base choice , subject to parameterization and sensitivity analysis .

| Appliance | Typical number | Energy usage/unit | Emission level/unit |
| --- | --- | --- | --- |
| Stove | 1 | 600 W , 1 h /day |  |
| Microwave oven | 1 | 300 W , 1 h/day |  |
| Freezer | 1 | 250 W , constant |  |
| Washing machine | 1 | 500 W , 100 W standby , 2 x 2 h/week |  |
| Dish washer | 1 | 500 W , 100 W standby , 3 x 2 h /week |  |
| TV | 1 | 130 W / standby 6 W , 3 h / day |  |
| DVD | 1 | 200 W , 2 h/day |  |
| PC | 2 | 300 W , 50 W standby , 5 h/day |  |
| Light bulbs | 10 | 5 W/m2 , 15 h/day |  |
| Spot lights | 3 | 1 W/m2 , 3 h/day |  |

Table 4: Appliance consumption and usage (Source: ww.sbi.dk Report Sbi 2006:06)

The UK Energy labelling directive for Sustainable products 2006 contains some tables SP06 for energy consumption of some appliances, but does not state as in Table 3 the average usage time.

8. EQUIPMENT ALTERNATIVES w.r.t ENERGY SOURCES

Thanks to the quantized control levels explained in Section 5.3, this research gains another degree of freedom in exploring emission optimization .More specifically, as the project will use the output levels in a given flow, vs. discretized control levels, nothing is assumed explicitely about the energy sources fueling these equipments! For each control level, each equipment will have, for any possible realistic energy source, output level and energy source consumption

Figure 11: Equipment or appliance plant model

This means that , when considering choices between equipments or appliances, there will always be a range of options for given equipment type, between fuel types and peak output levels .The optimization then determines between the range of options , which are the best in associations between equipments .

Figure 12: Equipment or appliance choice by category, peak output value and energy source

9. EMISSIONS CONTROL BOX

9.1. Functional architecture

As explained in Section 3, the research should allow designing a closed loop control version of emissions at house level embodied in an emissions control box. Subject to interoperability constraints , such an emissions control box would receive all relevant sensor inputs in the house, and determine via algorithms the time-dependent load controls sent to the equipment specific decentralized controllers so that ,in a predictive and coordinated way, the overall house emissions be reduced .Subject to off-line computations ,and sufficient storage capacity in the emissions control box, it could be powered by a commercial PLC or embedded processor (like PPC architecture) .

In the diagram Figure 13 below, the generic designation Load (U) refers to the 1-day or 1-year Load levels applicable to control variable U, in that U is specific to each equipment .The quantization of the Load levels is explained in Section 5.

Figure 13: Basic architecture of emissions control box; labels (i,j) refer to house volumes and t to clock

9.2 Computing and Communications infrastructure

- In the context of the closed-loop emissions optimization in this project , computational demand will probably exceed the microcontroller processing power found (at best)  in most local equipment controllers .Therefore , in the present project, will be investigated the feasibility of a micro-grid of all home based embedded processors (e.g. Power PC architecture or equivalent) and microcontrollers , linked together by grid protocols . Not only will this idea allow reducing the cost of the emissions control box, but it will by the same token give extended processing power to the individual processors or microcontrollers when needed.

- The base communications architecture in this project are wireless links between sensors, controllers and he emission control box. In this way housing refurbishment and systems installation costs will be minimized .Whereas most research has dealt with wireless sensor networks, this project will enable wireless sensors and controllers to play together with the emissions control box as a possible hub .The selection of the transmission technology (ZigBee, Bluetooth, wireless Ethernet, etc) will be made during the course of the project.

10. EMISSION PERFORMANCE MEASURES

This Section does not propose criterion functions or a criterion function hierarchy as discussed later in Section 11. It aims only at setting such criterion functions in view of macroscopic emissions policies.

Some groups favour as performance indicators across a sector such as the housing sector, the notion of emission intensity .It consists in calculating, in a given sector and at a given moment, the emission per unit of value added in the sector .In this way the evolution of the emissions gets weighed by the importance of the sector in the total emission.

The EU wants to reduce emissions from buildings by a factor of four before 2050.According to CITEPA; housing represents 20 % of emissions in Europe (behind transports 27 % and industry 21 %). The French PREBAT project aims at average energy consumption in housing of 50 kWh/m-2/year (against present 160-300 kWh/m-2/year)

The IPCC is evaluating the best scenario figure of 445 parts per million of CO2 in the atmosphere, while EU has been pushing for a 2 degrees Celsius cap on temperature increase.

Such policies set some scales of gains to be achieved by optimization in this research.

11. EMISSIONS ANALYSIS

11.1. Energy data

The definitive source, for UK housing, is the Domestic Energy Fact File. Both the 2003 issue and the 2006 issue (which breaks the information down by tenure) are available at: http://www.bre.co.uk/housing/page.jsp?id=396

It contains a huge amount of data about domestic energy consumption for all purposes, including heating and hot water using different fuels. I attach a succinct summary in energy units - it is a simple matter to convert them into emissions using the standard carbon intensity factors for the different fuels.

11.2 Emissions

While initially the research, for lack of better emission data, shall use the emission equivalents of the emissions for the heating and cooling in a home on the basis of energy balances (see Appendix I and Section 2), a second stage in the project much refine this analysis .Emissions of air pollutants from the combustion process in gas and oil fired boilers are:

-carbon dioxyde CO2

-nitrogen oxydes NOx

-carbon monoxide CO

-methane CH4

-sulphur oxides SO2, volatile organic compounds CxHy, and particulate matters in oil fired boilers only

For different boilers VHK has indicated the following default values per heat generator type (Table 5):

| HEATING : emissions to air per GJ of heat output | Primary energy MJ | GWP kg | AD g | VOC mg | HM mg | PM g | To water HM mg |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Electric, per GJ | 3045 | 132 | 784 | 1147 | 180 | 17 | 6 |
| Gas , 86 % atmospheric | 1163 | 64 | 19 | 846 |  |  |  |
| Gas , 90 % atmospheric | 1111 | 61 | 18 | 809 |  |  |  |
| Gas, 101 % condensation | 990 | 54 | 16 | 721 |  |  |  |
| Gas, 103 % condensation | 971 | 53 | 16 | 706 |  |  |  |
| Oil , 85 % atmospheric | 1176 | 87 | 110 | 1519 |  | 2 |  |
| Oil , 95 % condensation | 1053 | 78 | 98 | 1360 |  | 2 |  |

GWP: global warming potential (Co2, CO, NOx)

AD: acidification potential (SOx, NOx)

VOC: volatile organic compounds (CxHy)

HM: heavy metals toxicity incl. CO

PM: particulate maters

Table 5: Emissions to air per GJ of heat output, for different types of emissions

The implication is that the goal functions will change according to the emission goals and involve a mix of specific emissions.

12. EMISSION AND COST OPTIMIZATION CRITERIA

12.1 Emissions goal functions

When addressing the optimization of emissions in homes, several analysis levels coexist and interact:

-the very short term effects linked to the device controllers around chosen stabilization target levels ;an example thereof could be a radiator thermostat ; such a level is not addressed in the project

-the daily usage cycle linked to human usage and activity in the house , itself subject to seasonal variations ; an example thereof is the possible reduction of heating and lighting in unoccupied rooms

-the annual cycle linked mostly to the selection of equipments, materials, appliances and architectural design; an example thereof are the possible substitution effects between energy equipments for smaller emission levels

| EMISSIONS OPTIMIZATION PROBLEM | Time horizon | Specification | Data and control sampling rate |
| --- | --- | --- | --- |
| Daily usage cycle | 1 day | Average emissions per day over 12 days of the year spread over 12 months | 1 minute |
| Annual cycle | 1 year | Integrated emissions over the year | 1 day |

Table 6: The daily usage and annual emissions optimization problems

The project will focus on the daily usage cycle and the annual cycle, in optimizing cumulated emissions over these time intervals as specified in Table 6 above. For each problem, the controls are getting jointly optimized as explained in Sections 3-5, with data from sensors and controls sampled at the stated intervals.

12.2 Cost analysis

While the emphasis and optimization will deal with the emissions , possibly categorized as explained in Section 11,  it is essential to determine the corresponding amortized investment and operational costs derived from the optimal emission profiles .In other words, for each set of time dependent profiles of controls (over a day or over a year) which each minimize emissions over these time intervals , the project shall compute as a sub-product the resulting operating costs and investment levels amortized over said time intervals . Such cost based validation is necessary as it is likely that conflicts will occur between the sixteen multiple criteria situations depicted below:

-Low daily emissions/Low annual emissions/Low daily costs/ Low annual costs (ideal situation)

-High daily emissions / High annual emissions / High daily costs / High annual costs (worst situation)

-Low daily emissions / High annual emissions / Low daily costs / Low annual costs (unlikely but maybe linked to bad heating equipment in winter)

-High daily emissions / Low annual emissions / Low daily costs / Low annual costs (unlikely but possibly linked to wasteful usage of appliances)

-Low daily emissions / Low annual emissions / High daily costs / Low annual costs (unlikely but possibly linked to excessive usage of electrical heating but for a seasonal home usage)

-Low daily emissions / Low annual emissions / Low daily costs / High annual costs (linked to expensive although emission efficient equipments, and bad home designs)

-Low daily emissions/ Low annual emissions / High daily costs / High annual costs (linked to expensive although emission efficient equipments and appliances and excessive usage)

-etc …

One unique strength of the proposed methodology is to be able, for the same home and equipment model, and with the same usage assumptions, to analyze separately and jointly all these alternatives, using possibly multiple criteria decision making and game theoretical Equilibria to identify rigorous compromises.

Regarding amortization parameters, it will be assumed that all house equipments will have a life time of 15 years, while appliances will have a life-time of 5 years; if needed in the annual emissions problem, the nominal rate of depreciation shall be 3, 5 %.

13. OPTIMIZATION ALGORITHMIC APPROACHES IN THE PROJECT

They can be summarized by the attached Table 7:

| MODELLING LEVEL | METHODS | JUSTIFICATION |
| --- | --- | --- |
| Top level | Combinatorics | The housing context , equipment choices, fuel choices ,and housing design all are discrete choices addressed by combinatorics |
| Medium level | Finite elements in time and space | Time sampling is imposed by boiler operations cycles, as well as housing usage. Volumetric sampling is imposed by the interactions between different volumes in different rooms, including walls and separations. |
| Lowest level | Min Max dynamic optimal control | Taking dynamics in a given time interval and a given volume into account, the maximization of emissions deals with random occurrences due to surroundings , usage (water, people, lighting etc), etc under these time and space assumptions. The resulting minimization of emissions is for the dynamic controls applicable to the system. Possibly a cost term to be minimized also would have to do with the thermal comfort for users by minimizing variance from some usage dependent target values |

Table 7: Optimization approach hierarchy in the project

Furthermore, with reference to Section 2, 3, 4 and 11, some computational issues can be identified (those linked to constraints are further discussed in Section 14 and those linked to random factors in Section 15):

-all optimizations will be with explicit constraints set by the building regulations and standards ;most such constraints will be on state variables, which complicates a lot the numerical resolution in dynamic situations

-all optimizations should be able  , over long time intervals, to satisfy static cost constraints as explained in Section 14 ;the cost ceiling values must be overall design parameters ; if this proves too complicated, cost consequences alone will be quantified

-the cross-impact nature of emissions, as opposed to energy , as explained in Section 11, will in a later phase pose key problems of controllability for a chosen simple set of control variables; one method is to linearize each problem to be solved, and to analyse the controllability matrices to identify insufficient controls.

-uncertainties modelling will probably have to be simplified to some extent just due to the computational load.

More work is needed to determine if and how the emissions optimization goal functions, as opposed to just having constraints, should include or not elements representing a balance with a thermal comfort for humans. Said otherwise, the best emissions regime over time in a home may be far from comfortable due to variations / peaks in temperature, humidity, light levels etc.

It is necessary to point out that a specific optimization run will depend on initial and terminal conditions over the chosen time interval. This is especially crucial for emissions, and different from energy optimization as:

-on one hand, the start-up of equipments in the house with use of energy will generate a disproportionately higher quantity of emissions

-the planned turning off of equipments using predictive average information about home and appliance use, together with the explicit account for time constants, will allow to reduce emissions earlier and thus to reduce their quantity.

14. OPTIMIZATION CONSTRAINTS

14.1 Thermal comfort constraint

By optimization constraints are meant a set of quantified conditions (inequalities or equalities) which any solution to the optimization must satisfy the optimization of the emissions must satisfy heat comfort constraints as indicated in Section 2. The legal UK constraint is that the indoor temperature should at least me 16 degrees C in occupied rooms. The optimizations will be carried out with such a minimum temperature condition, which can be set as a parameter, with 16 degrees as default value. It would be preferable to specify in which parts of the stock housing rooms this condition should be met.

14.2 Capital budget constraint

As an alternative to multiple criteria emissions optimization and cost analysis as explained in Section 11.2, one can run the emissions optimization problems with, in addition to the thermal comfort constraint above, a capital budget constraint . This would mean that any annual emissions optimal design would have to give a room temperature above a given threshold, while at the same time costing on an annual basis less than 2500-3000 GBP in capital budgets. In this figure it is assumed that the boiler contribution would represent approximately 1800 GBP.

15. RANDOM FACTORS

Subject to much higher computational complexity, and subject also on assumptions on such uncertainties (such as a probability distribution, or characteristics thereof under simplifying assumptions), it should be possible to study the robustness of the optimal emissions designs.

The key uncertainties to be addressed include:

a) service usage demands by occupants (see Section 6 and Figure 4), in adding behavioral uncertainties about the users’ demands; as statistical distributions may be subject to caution, the best option here is to determine some extreme situations and assume uniformity in-between

b) weather: good weather distributions exist for temperature and illumination; any specific choice would assume that a default location in UK be chosen to choose the corresponding weather distributions if available for that location

c) materials: by manufacturing, but mostly due to installation and aging, there is quite variability on the heat transfer parameters; such distributions may be hard to find but some experience data may be used

Regarding the weather and season , SEDBUK (3Seasonal efficiency of domestic boilers in the UK”) is the recommended measure of the seasonal efficiency of a boiler installed in  typical domestic conditions in the UK ,and it is used in SAP assessments and the Building regulations .SEDBUK efficiencies of the most current and obsolete boilers can be found on www.boilers.org.uk .

16. SOME INNOVATIVE TECHNOLOGIES AND OTHER RELATED RESEARCH AREAS

The references in Section 17 point at many innovations and a few disruptive ones in the field of energy control or emissions .This research project should definitely emphasize early modelling around disruptive technologies, to embed corresponding functionalities or parameters. The text below just gives some examples of such disruptive technologies and where they can be identified early enough for impact on this research project.

16.1 Transmission and person detection technologies

The 2002  upstart WIRECOM Technologies, Orléans(F) uses power line carrier communications to manage energy control in buildings ;it has 700 installed sites and claims a payback period of 4 years max. It also has person detection sensors, shutting down light, and ventilation while heating is reduced .The starter product for individual homes costs 1000 euros.

16.2 Wireless sensor and actuator networks

The area of wireless sensors networks is a very active one in the research community. Less so is the field of wireless actuators which offers challenges of its own because of synchronization of control policies and their integration with overall home controls. This project will certainly lead to interesting research topics allowing to bring further to the fixed equipment controllers as well as the appliances some usage dependent and emissions minimizing capabilities. The Finnish company Vallox Oy already offers wireless controllers for ventilation systems.

16.3 Sensors

The University of California Institute of energy site in Berkeley embeds sensors into lighting systems to reduce energy consumption

16.4 Energy efficiency computer tools

Such tools are developped by many small companies. Amongst those with multinational operations are: BuildDesk (with the acquisition of Dutch CEA Group –MD Johan Gijsman), ERGELIS (which does remote control for hotels and companies based on person counters)

16.5 Utilities emission levels

The German utility company EnBw Energie Baden Würtenberg AG claims to have he lowest CO2 emissions in Germany amongst all utilities (www.enbw.com)

16.6 Major events displaying new technologies

The French Assises nationales de l’énergie hosts each year over 1000 people interested in the subject of the energy efficiency of buildings (www.assises-energie.net). ADEME also runs the Programme de recherche et d’expérimentation de l’énergie dans les bâtiments (PREBAT), which has an annual 15 M Euro funding.

17. REFERENCES

The emphasis on industry sources.

1.    UK Government Market transformation Web site  http://www.mtprog.com/ : Briefing notes , Cross      Sector reports , XS05 heat replacement effect , XS 24 overview of developments , XS 29 : simulation of lighting and heating
2.     Domestic heating and hot water briefing notes – especially BNDH19
http://www.mtprog.com/ApprovedBriefingNotes/PDF/MTP_BNDH19_2007March30.pdf ; 
it seems university of Strathclyde has a simulation model referenced in
BNDH19. One of the best UK academic research groups appears to be
3.     University of Strathclyde http://www.esru.strath.ac.uk/
4      EPSRC funded project that considers human occupancy of buildings
  http://gow.epsrc.ac.uk/ViewGrant.aspx?GrantRef=EP/C517520/1
5.      ISO/CEN TC247 on Building Automation Controls.
6.     CACHE TRC, Process Control Process Control and Optimization Consortium Process Control Power Point slides by Dr. James B. Riggs, Texas...

http://www.che.utexas.edu/cache/trc/t_process.html

7.	ERC Fireside Performance Optimization

... Power generation and environmental control to provide solutions to global ... performance monitoring and optimization and emissions control, with emphasis on...

http://www.lehigh.edu/energy/workshop_shortcourse/shortcourse_fireside.htm

8.	ERC Fireside Performance Optimization

This short course covers fireside performance monitoring and optimization and emissions control, with emphasis on the latest developments in techniques,

http://www.lehigh.edu/~inenr/workshop_shortcourse/shortcourse_fireside.htm

9.	Boiler Control System Optimization 	... and reduced emissions. VALUE Boiler control system optimization contributes to improved plant efficiency. 	http://www.epriweb.com/public/000000000001011191.pdf

10.	Multisimplex, software, optimization, process, control, automation...

Grabitech works with process optimizing, process, optimizing, multisimplex, emission minimization, wonderware, camo, software, optimization, spc, process control, 	http://www.grabitech.se/

11.	Technology Organizations - Electronics & Energy Conservation...

The lab develops control and optimization algorithms to minimize fuel, enhance ... engineering with core focus on process controls, estimation and optimization. ...

http://www.ge.com/research/grc_4_4_6.html

12.	EPA praises Koch for emissions-control programs - Wichita Business...

EPA praises Koch for emissions-control programs,

http://www.bizjournals.com/wichita/stories/2000/07/24/daily22.html

13.	Implications 	... that stringent measures to control emissions of CO 2 would be ... that much stronger cuts in emissions are defensible on economic ... conclusions obtained from optimization models are very sensitive ...http://stephenschneider.stanford.edu/Climate/Climate_Policy/PolicyImplications.html

14.	A holistic approach to process optimization and emissions control...

Honeywell Europe's Paul Brice talks to Peter Welander about their offerings for process design, simulation, optimization, and emissions control using...

http://www.controleng.com/index.asp?layout=audio&element_id=2140035564

15.	MultiSimplex AB 	Experimental design and multicriteria optimization software, based on fuzzy logic and the simplex method for sequential design of experiments. 	http://www.multisimplex.com/

16.	RAINS - Basics of the RAINS Model

A free and competitive market for the exchange of emission control technology is ... The optimization module identifies, for given set of regional target...

http://www.iiasa.ac.at/Research/TAP/rains_europe/intro.html

17.	MIT ChE: Research - Energy and Environmental Engineering

... Edu combustion science, engineering, and emissions control; applied chemical kinetics; flame and ... ... process modelling, numerical analysis and optimization Kenneth A. Smith Professor 617.253.1973 kas ...http://web.mit.edu/cheme/research/energy.html

18.	Emission Control Technologies

Energy Conversion System Optimization. Environmental Control Technologies ... the development of control technologies to reduce emissions from mobile and...

http://www.undeerc.org/centersofexcellence/emission.asp

19.	Getting "Smart" about Optimization

Control to ensure emissions don't venture beyond. acceptable limits. ... With the controls and. automation software, steam temperature swings, which...

http://www.emersonprocess-powerwater.com/news/ae/pe-smartprocess.pdf

20.	Home 	emissions for optimization of emissions control, design and install complete systems. Services Include: Air Pollution Control Systems: Air Emissions Testing Services

http://tracetechnologies.biz/AboutTrace.html

21.	Optimization of Product Life Cycles to Reduce Greenhouse Gas Emissions...

Control Units, CRT Monitors, and LCDs. optimization approach evaluates the potential to reduce GHG emissions at each of...

http://www.energy.ca.gov/2005publications/CEC-500-2005-110/CEC-500-2005-110-F.PDF

22.	Press Release 	... Storage Applications and Technologies) October 2005, San Francisco, CA Process Optimization as a Retrofit Emissions Control Strategy Infocast Meeting Washington DC, October 5, 2005 Windy Expectations ... 	http://www.pearlstreetinc.com/press_release.htm

23.	Altech Environmental Consulting - Engineering & Technical Services

... and innovation in the areas of contaminant control, pollution prevention and process optimization. ... Air Emissions Control. We provide monitoring and control services in this...

http://www.altech-group.com/aecl_engineering.htm

24.	Honeywell Emissions Control FAQs 	FAQs about Honeywell Emissions Control...	http://hpsweb.honeywell.com/Cultures/en-US/IndustrySolutions/Power/DistrictHeating/EmissionsControl/FAQ/default.htm

25.	Control Engineering Podcasts - 2/7/2007 9:50:00 AM - Control...

Honeywell Europe's Paul Brice talks to Peter Welander about their offerings for process design, simulation, optimization, and emissions control using Honeywell's suite of capabilities with UOP...

http://www.controleng.com/article/CA6414326.html

26.	Integration of Reliability, Safety and Security 	... final control elements, and key software pieces (such as optimization software, combustion control systems in the boiler, as well as emissions controls like 	http://www.isa.org/~powid/powid_2006/Programlong20060509rsd3a.htm

27.	Service Line | Plant Automation Services | Process Optimization Ability to execute turnkey projects in advanced controls ... for developing applications in the field of Process Control and Optimization...

http://www.tcs.com/eis/ServiceLines/vcoshow.aspx?sname=PlantAutomation&index=775

28.	RealClimate » Buying a stairway to heaven?  Heaven? Filed under: Climate Science Greenhouse gases _ Just in the last year or so, a new type of scheme for reducing personal carbon emissions has appeared, the remarkably painless ...	http://www.realclimate.org/index.php/archives/2006/05/buying-a-stairway-to-heaven/

29.	Energy & Utilities Performance Management System control...

Energy & Utilities Performance Management System control optimization oil ... and heat recovery energy and capital Pinch analysis capital costs, emissions,	http://www.lightridgeresources.com/

30.	Global optimization of climate control problems using evolutionary and...

Global optimization of climate control. Problems using evolutionary and stochastic ... optimal control problem of choosing the optimal greenhouse gas emissions...

http://www.geosc.psu.edu/~kkeller/Moles_asc_02.pdf

31.	Online Optimization 	... and production control to maximize profit and minimize emissions by providing ... optimization, and sends the optimal set points to the distributed control system...

http://www.mpri.lsu.edu/iooindex.html

32.	Solving nonconvex climate control problems: pitfalls and algorithm...

... Control problem of choosing the optimal greenhouse gas emissions ... nonlinear optimal control (dynamic optimization) and. parameter estimation problems. ...

http://www.geosc.psu.edu/~kkeller/Moles_et_al_asc_04.pdf

33.	Neural Network Optimization of Boiler Emissions Control...

Amazon.com: Neural Network Optimization of Boiler Emissions Control with Gas Reburn:

http://www.amazon.com/Neural-Network-Optimization-Emissions-Control/dp/B0006R7THE

34.	Optimization of Product Life Cycles to Reduce Greenhouse Gas Emissions...

... life-cycle optimization considers the reduction of energy use ... energy-efficient control units and displays, and the use of LCDs instead of CRT monitors...

http://www.energy.ca.gov/2005publications/CEC-500-2005-110/CEC-500-2005-110-D.PDF

35.	TAF Peer Review: Emissions/Cost Modules

Optimization models can be used to assess the cost reductions due to the Title IV ... planning horizon results in significantly lower costs and different emissions control...

http://www.lumina.com/taf/taflist/tprcost.html

36.	APC-network.com 	... provider of Advanced Control, Production Optimization and Performance Monitoring, and simulation ... ... organisations reduce energy costs, control emissions and improve process performance ...	http://www.apc-network.com/apc/Default.aspx?tabindex=2&tabid=29

37.	Green Affordable Housing Coalition. sheet topics, including HVAC Optimization, Sizing Water Heaters, Ventilation, and more. ... As, asthma, mold control, carbon monoxide, lead, backdrafting,

http://www.frontierassoc.net/greenaffordablehousing/FactSheets/FactSheets-Technical.shtml

38.	Energy Systems - Business White Papers, Webcasts and Case Studies - BNET.com

... System With ABT Optimization Engine KALKI ... the European Union Emissions Trading Scheme IEA ... the Use of Concrete in Housing Cement Concrete & ... Slab Heating & Cooling Control BNP Media October...

http://industries.bnet.com/ENGINEERING/Mechanical+and+Industrial/Energy+Systems/

39.	Rosemount Analytical - Gas Division Home ... Gas Analyzer series helps you control your process by first defining the ... three versions (general purpose, field housing and flameproof) so there's one ...

http://www.emersonprocess.com/raihome/gas/default.asp

40.	Acronyms. s Mate EMCON Emissions Control EMD ... FOT Force Optimization and Training FOT&E ... GFCS Gun Fire Control System GFE Government ... System HMIS Housing Management Information ...	http://www.uscg.mil/ccs/cit/cim/AcronymDictionary.htm

41.	Case Studies 	... area of the US Postal Service required an improvement in energy efficiency, improved emissions control ... College officials desired a new facility that incorporated a design for a centralized housing ...	http://www.conedsolutions.com/Business/Energy+Optimization/Case+Studies/

43.	IIASA 	Ministry of Housing, Spatial Planning and the Environment ... Although there is a large variety of an option to control emissions, an integrated assessment...

http://www.iiasa.ac.at/~rains/reports/tfiam22a.pdf

44.	Environmental Valuation & Cost-Benefit News ... strategy and arms control, energy and ... efficient than private housing. A summary document ... of making emissions reductions is the ... Transport Modelling Optimization The Department of ... 	http://www.envirovaluation.org/

45.	Sitemap ... Energy & Fuels Emissions Ventilation & ... Power Construction & Housing Det Norske Vegvesen ... mobile test system to control odour Mobile ... PolyScan for optimization of flexible ...

http://www.force.dk/en/Header/Sitemap/

46.	hcl-en-engl.PDF  ... one housing. Two separate regulating outputs. Control structure: ON/OFF, P, PI, PID - heating; 	http://www.paulgothe.de/english/inhalt/acid.pdf

47.	Texas-home-insurance - Company Home Houston Insurance Owner

... Building floating housing bound for New Utopia ... GPS surveys, Geodetic Control Networks, and GIS ... search engine optimization with no advance ... decreases emissions while ... Houston...

http://texas-home- insurance.homelifehealthautocarinsurance.info/companyhomehoustoninsuranceowner/

48.	Facilities Management Calendar of Events	... for greenhouse gas emissions and waste. These issues are ... ARDA), and National Multi Housing Council (NMH) 2006 Conference ... requirements: applications and control. For more information visit ...http://www.fmlink.com/News/Events/

49.	out-stack-staub-engl.PDF ... one housing. Two separate regulating outputs. Control structure: ON/OFF, P, PI, PID - heating; 	http://www.paulgothe.de/english/inhalt/dust-out.pdf

50.	pdc's bookmarks tagged with "housing" on del.icio.us 	5 carbon-emissions. 1 case-study. 1 ... 1 source-code-control. 2 sourceforge.  .http://del.icio.us/pdc/housing

51.	Outline for Background Paper (Draft of entire Background Paper ...	... industrial gases that also need to be curbed to control GHG emissions. ... Agency, the Business, Transportation and Housing Agency ... Optimization of these practices could enhance the capture...

http://www.ciwmb.ca.gov/agendas/mtgdocs/2007/05/00021945.doc

52.	Task 28 Sustainable Solar Housing	natural cooling and solar/glare control. Task 28/38 will address cost optimization of the mix of concepts reducing energy ...	http://www.iea-shc.org/task28/index.html

53	AEC - Site-wide News	... GSA office building housing the US Department of ... and VAV system optimization, laboratory ... improved economizer control, system rebalancing ... greenhouse gas emissions by enabling new ...	http://www.archenergy.com/aboutaec/news/

54.

55.	Renewable Energy, Energy Research and Development, CANMET Energy ...	Modelling, Simulation and Optimization ... Integrated Emissions Control ... Housing, Buildings and Simulation: Other Housing, Buildings and ...http://www.nrcan.gc.ca/es/etb/cetc/cetc01/htmldocs/Publications/

56.	schwerm-engl.PDF 	... one housing. Two separate regulating outputs. Control structure: ON/OFF, P, PI, PID - heating;	http://www.paulgothe.de/english/inhalt/heavymetal.pdf

57.	What We Do 	We do this through planning, designing and managing the construction of housing? ... such as energy conservation, optimization of site set-and-finish...

http://www.hickoryconsortium.org/1_what/whatWeDo.htm

58.	TRB Publications Index Sitemap

... reliability of speed control system evaluations ... models work zone optimization for multiple lane ... logit model for dutch housing choice validation of ... gas and particulate emissions by driver and road ...http://pubsindex.trb.org/sitemapCrawl.asp

59.	Mitsubishi go twin turbo ... The short throw speed control and high strength ... its tall rear axle housing and a VGA cable is ... The EPA exhaust emissions regulations. It ... search engine optimization axle housing and ...http://2998.9nk4ui5o.info/

60.	CH2M HILL 303.771.0900 	... pollutant reduction, motor vehicle and industrial emissions control ... Assessments (NIA) for town planning, public/private housing ... solutions in acoustic design as well as noise barrier optimization ...	http://www.ch2m.com/china2/english/Experience/hongkong.htm

61.	DRAFT ENVIRONMENTAL ASSESSMENT TOXECON RETROFIT FOR MERCURY AND MULTI ...	integrated emissions control system proposed by We Energies and its project partners with cost ... Housing data for Marquette, Ishpeming, Negaunee and ...

http://www.netl.doe.gov/publications/others/nepa/draft_ea_WEPCO.pdf

62.	Renewable Energy, Energy Research and Development, CANMET Energy...

Advanced Controls, Simulation and Emissions ... Other Housing, Buildings and Simulation publications ...	http://www.nrcan.gc.ca/es/etb/cetc/cetc01/htmldocs/Publications/index_e.htm

63.	Social Edge - Creating a Hybrid For-Profit / Non-Profit Social Enterprise St...

... We practice what we preach (use of CSR/CSI optimization in a profit driven business structure) (We ... advantage as well as non profit advantage from housing the two structures together. - Many...

http://www.socialedge.org/?128@290.yVPSaJHgXzz.0@.3c432e32

64.	2004 GMRC Technical Papers by Title	Finally, this paper will summarize the emissions control effectiveness ... the development program is a catalyst and converter housing ... Next the optimization of the compressor stations is formulated as ...	http://www.gmrc.org/index-title.php?year=2004

65.	ATSDR - PHA - Norfolk Naval Shipyard, Portsmouth, Virginia 	... air emissions is documented in a 1973 air pollution control study conducted at NNSY (NNSY 1973) ... housing units and soils, advised of simple housekeeping ...http://www.atsdr.cdc.gov/HAC/PHA/norfolkshipyard/nns_p2.html

66.	Task 28 Sustainable Solar Housing 	... consumption for heating and cooling, and minimal CO2 emissions. ... Day lighting (for improved living quality) and - natural cooling and solar/glare control. ...

http://www.iea-shc.org/task28/

67.	Building Energy Software Tools Directory: Tools Listed Alphabetically

... Validation; solar control, overshadowing ... life cycle costing, emissions impacts, alternative ... programming, optimization methods, HVAC ... GIHMS industrialized housing production operations http://www.eere.energy.gov/buildings/tools_directory/alpha_list.cfm

68.	Headlines/Breaking News from The Business Journal of Portland -...

... Provides funds for affordable housing projects, celebrated its 10-year anniversary by announcing ... 25 million on new emissions-control...http://www.bizjournals.com/site_map/portland_sitemap_84.html

69.	Low Noise, increased reliability, digital control - a new...

Lower both, the noise emission and the high and low pressure pulsation the ... Figure 11: Optimization of suction path in housing and control journal in...

http://www.moog.com/media/1/LowNoiseNewGenRKP-TechPaper.pdf

70.	Environment | CMHC... reduction of development impact. Optimization of new ... 4.2 Sediment and Erosion Control of Construction ... with the construction and operation of housing. (Note that Green House Gas (GHG) emissions are ...	http://www.cmhc-schl.gc.ca/en/inpr/su/eqho/eqho_005.cfm

71.	FCCC/SBSTA/2002/INF.1 	F-gas emissions as a result of measures related to the housing support schemes ... in carbon consumption, better emissions control systems and continuous ...

http://unfccc.int/resource/docs/2002/sbsta/inf01.pdf

72.	Products – Oil Fired Combustions Optimization - Storm Technologies...

Combustion optimization when firing residual fuel oil is important to control stack particulate emissions, boiler gas flow ... Fuel Line Orifice Housing: Oil Fired Combustion Optimization: STORM ® .http://www.stormeng.com/OilFiredCombustion.php

73.	A Coupled Bottom-Up / Top-Down Model for GHG Abatement Scenarios in the...

Emissions by the housing sector declined from 12.4 Mt in 1991 ... optimization of the energy sector only since both the supply options and...

http://ecolu-info.unige.ch/~nccrwp4/GEMINI-E3/GEMINI_ETEM.pdf

74.	CAEP WORK PROGRAMME LEADING TO CAEP/7 1. NOISE 1.1 CURRENT TECHNICAL ... analyses population/housing developments around airports ... various emissions and noise, the likely timescale for introductions and appropriate inputs for...

http://www.icao.int/icao/en/env/WorkProgramme_Caep6.pdf

75.	Sustainable Development 	housing, schools, hospitals and large infrastructure projects that attest to the ... 8.-To measure and control entries, processes, products. and emissions...

http://www.holcim.com/gc/MX/uploads/Sustainable Development.pdf

76.	A guide to offsetting your carbon emissions | By Erica Gies | Grist | Main D...

... and traffic-signal optimization. Beyond carbon ... such as solar for a housing project in Chicago ... is used to gauge CO2 emissions for driving and ... food movement.

http://www.grist.org/news/maindish/2006/10/10/gies/

77.	Chattanooga Housing Authority and Honeywell Reduce Energy Emissions...

Chattanooga Housing Authority and Honeywell Reduce Energy Emissions and Annual Costs by $1.25 Million...	http://www.tmcnet.com/usubmit/2005/May/1143906.htm

78.	Flex Your Power: Hot Topics: Congratulations, Energy Efficiency Leaders

... s four affordable housing apartment buildings ... class monitoring and control applications. Using ... greenhouse gas emissions 11% per sq. ft ... annually through optimization of the heating...

http://fypower.com/feature/congrats/

79.	POWER-GEN International 2006	Housing Information - Online registration closed. Housing Form (PDF) Travel Visa Details ... Addressed will be fuel, equipment and system modifications for the control and optimization of emissions and other...

http://pgi06.events.pennnet.com/fl/content.cfm?Navid=4851&Language=

80.	http://www.wortschatz.uni-leipzig.de/Papers/top10000en.txt . engine III conducted messages housing entry knowledge England We're ... responsibility Series traded Control sort disease MS-DOS ... prominent Hollywood emissions ultimately two-year summary ...http://www.wortschatz.uni-leipzig.de/Papers/top10000en.txt

81.	Energy Task Force Resources - Energy Efficiency & Conservation

... for the Virginia Department of Housing and Community Development ... of aerospace vehicles, optimization of propulsion systems, and ... cruise control, designed to reduce emissions and fuel ...

http://www.research.vt.edu/energy/reseff.html

82.	Keeping you current with Industrial Heating Equipment Association

Leslie Muck, Website & Housing/Registration Manager Leslie works closely with ... The Center for Emissions Control, RadTech International,

http://www.ihea.org/images/IHEA%20VolXXIX%20No2.pdf

- “Assessment of the size and composition of the UK appliance population”, UK Dept Trade and industry, Report URN 05/1942, 10 Nov 2005

- H. Erbe, V. Sguev, et al , Eds, Proceedings IFAC Workshop on Energy saving control in plants and buildings, Bansko, Bulgaria , 02-05 Oct 2006, Publ. by Pergamon Press

- Title: Building regulation and sustainable housing. Part 1: a critique of Part L of the Building Regulations 1995 for England and Wales Author(s): Malcolm Bell, Robert Lowe Journal: Structural Survey ISSN: 0263-080X Year: 2000 Volume: 18 Issue: 1 Page: 28 - 37 DOI: 10.1108/02630800010322517 Publisher: MCB UP Ltd Abstract: This paper presents a critique of the energy efficiency aspects of the Building Regulations for England and Wales as they relate to dwellings. It is argued that a significant improvement in the regulations is required if large reductions in CO2 emissions are to be achieved in the first half of the next century and that considerable scope for regulation improvement exists (almost 90 per cent reductions in space heating are achievable). The paper identifies the shortcomings of the methods used to demonstrate compliance and proposes new approaches which would increase consistency and enable a clear programme of improvement to be established. Keywords: Building regulations, Efficiency, Energy, Housing Article Type: General review Article URL: http://www.emeraldinsight.com/10.1108/02630800010322517

- DEFRA, Energy, cost and carbon savings calculations for the draft EEC 2008-11 Illustrative mix , Jan 2007, www.defra.gov.uk

- EMISSIONSTAT Global data base on emission data , Commercial , www.enerdata.fr

88.       Martin O’Hara, Philip Smith, Control schemes for OpenTherm boilers: energy efficiency and carbon savings in domestic heating, Danfoss Randall, Bedford, UK, 2007

89.       Martin O’Hara, Les Alexander , Thermostat control schemes for condensing boilers : energy efficiency and carbon savings in domestic heating , Danfoss Randall ,Bedford, UK , Jan 2007, (proprietary document)

90.        J. Cockroft, Aizaz Samuel, Paul Tuohy , Development of a methodology for the evaluation of domestic heating controls, DEFRA Market transformation programme project , Issue 1, ESRU , University of Strathclyde , March 2007

91.       ESP-r and the domestic controls interface, ESRU, University of Strathclyde, ftp://ftp.strath.ac.uk/Esu-public

92.       O.David, A.Fabre, Les économies d’énergie dans l’habitat existant (Energy savings in existing homes), Les Presses des Mines de Paris, Paris, 2007, 230 p.

93.       SAP: The Government’s standard assessment procedure for energy ratings of dwellings, 2001 edition www.housingenergy.org/uk

94.       Government’s housing energy efficiency best practice programme, General information leaflet no 59: Central heating system specifications (CheSS) Year 2002 www.housingenergy.org.uk

95.      BNXS 01: Carbon emission factors for UK energy use, Version 1.2, 19/04/2007 www.mtprog.com

APPENDIX I: CARBON INTENSITY FACTORS OF FUELS

As the basis in the research is initially on energy models, conversion to emissions impact has to be determined (see more in Section 11).

The carbon calculator provides a quick and simple estimate of the carbon footprints of individuals or households. The carbon footprint of an individual or household is the amount of carbon emissions generated as a result of the energy usage in heating, cooking, travelling etc.

The carbon footprint of and individual or household can be estimated. The carbon calculator calculates the carbon footprint by multiplying the amount of actual energy usage by an individual or household by the ‘carbon intensity factor’ of the fuels. ‘Carbon Intensity Factor’ of a fuel is the ‘amount of carbon emitted per unit’ of that fuel. For example, the carbon intensity of diesel is 2.63 kg CO2/liter as compared to O.19 kg CO2/kWh of natural gas. The carbon intensity factor of common fuels is given by the following Table 1:

| Fuel | Net Carbon Intensity Factor | kgC/kWh | Pence/kWh |  |
| --- | --- | --- | --- | --- |
| Natural gas | 0.19 kg CO2/kWh | 0.0518 | 2.90 |  |
| LPG | 0.21 kg CO2/kWh |  |  |  |
| Electricity | (*) 0.43 kg CO2/ kWh | 0.1175 | 10.00 (5.55 for water) |  |
| Heating oil | 0.27 kg CO2/kWh | 0.068 | 3.41 |  |
| Coal | 0.32 kg CO2/kWh | 0.0817 | 2.25 |  |
| Wood fuel (if made sustainable) | 0.0 kg CO2/kWh | 0.0068 | 1.60 |  |
| Sources: www.bp.com |  | [87] | [87] |  |

Table 1: Carbon intensity factors and costs  of common fuels ; (*) there is a lot of debate around this conversion factor ; the BP source uses 0,5460 kg CO2/kWh ; some use 0,53 kg CO2/kWh ;  the NEF and the Carbon Trust, plus OFGEM, all use the 0.43 conversion factor (see the calculator in the link below), this also gives the conversion factors for other fuel types; http://www.nef.org.uk/energyadvice/co2calculator.htm

The BNXS01 document [95] provides a procedure to use carbon emission factors from gas, electricity and oil, to construct MTP scenarios and policy briefs .Whereas there is agreement with Table 1 above on gas emission factors , that document has different data for electricity consumption 2007 ( 0,145 kgC/kWh) , electricity savings (0,141 kC/kWh) and domestic oil ( 0,072 kgC/kWh) .

APPENDIX II:  SURVEY OF UK HOME ENERGY AND EMISSIONS MODELS

The US Dept of Energy’s Energy efficiency and renewable energy program has compiled an international list of energy models for buildings (see below) .This full list is given in Appendix as many models deal with specific aspects needed in the present research, and are thus needed for reference ..A first table below gives only those models developped in the UK htp://www.eere.energy.gov/buildings/tools_directory/countries.cfm/pagename=countries/pagename_menu= united_kingdom

The links can be followed to access information on the specific models.

UK tools; Applications; Free; Recently Updated. Apache; thermal design, thermal analysis, energy simulation, dynamic simulation, system simulation. ApacheCalc; heat loss, heat gain, load calculation. ApacheHVAC; buildings, HVAC, simulation, energy performance. ApacheLoads; heat loss, heat gain, load calculations. ApacheSim; thermal simulation, energy consumption. Dataplus-online; monitoring and targeting, energy management, self-billing. DesignBuilder; Building energy simulation, visualisation, CO2 emissions, solar shading, natural ventilation, day lighting, comfort studies, CFD, HVAC simulation, pre-design, early-stage design, building energy code compliance checking, OpenGL EnergyPlus interface, building stock modelling, hourly weather data, heating and cooling equipment sizing. ECOTECT; environmental design, environmental analysis, conceptual design, validation; solar control, overshadowing, thermal design and analysis, heating and cooling loads, prevailing winds, natural and artificial lighting, life cycle assessment, life cycle costing, scheduling, geometric and statistical acoustic analysis. Energy Lens; energy management, half-hourly data analysis, business energy saving, monitoring and targeting. Envest; sustainable design, green buildings, life cycle analysis, environmental impact analysis. ESP-r; energy simulation, environmental performance, commercial buildings, residential buildings, visualisation, complex buildings and systems [91]. FLOVENT; airflow, heat transfer, simulation, HVAC, ventilation. FLUCS; illumination, day lighting. FlucsDL; daylight simulation. FlucsPro; luminaries, lighting design, lighting analysis, photometric data, radiosity. INDUS; ductwork sizing, ductwork design, HVAC. LifeCycle; life-cycle cost, economics. LightPro; luminaries, lighting analysis, photometric data. Microflo; CFD, airflow, air quality, thermal performance. Pisces; pipe work, heating, cooling. Radiance Control Panel; radiance, lighting, day lighting, ray tracing, glare. Radiance Interface; Lighting, day lighting, ray tracing, glare. SBEM; energy consumption/performance, carbon dioxide emissions, UK building regulations, compliance checking, non-residential buildings. ShadowFX; shading calculations, sun modelling, solar shading. solacalc; passive solar, house design, building design, building services, design tools. Solar Tool; overhang sizing and position, shading devices, louvers,. Suncast; solar shading, insulation. TAPS; pipe sizing. TAS; Building dynamic thermal simulation, building simulation , comfort, CFD, thermal analysis, energy simulation. Weather Tool; weather data visualization, psychometric, passive design analysis, optimum orientation, data synthesis..